# INDOOR AIR QUALITY SAMPLING

Robinson Hall Rowan University Glassboro, NJ 08028

PREPARED FOR:

Rowan University 201 Mullica Hill Road

Glassboro, New Jersey 08028

PREPARED BY:

Montrose Environmental, Inc. 500 Horizon Drive, Suite 540

Robbinsville, NJ 08691

MONTROSE PROJECT NO. 1200-62

### August 2023

TABLE OF CONTENTS

  - INTRODUCTION	1

  - PROJECT OBJECTIVE	1

- METHODOLOGY	2

- MOLD STANDARDS	3

- MOLD ASSESSMENT AND MOISTURE INPSECTION ACTIVITIES	4

  - VISUAL OBSERVATIONS	4

  - ANALYTICAL SAMPLING AND RESULTS	5

  - IDENTIFIED MOLD SPECIES	5

- CONCLUSIONS AND RECOMMENDATIONS	8

APPENDIX A

LABORATORY ANALYTICAL REPORTS

i

# INTRODUCTION

On August 23, 2023 Montrose Environmental, Inc. (Montrose) responded to a request by Rowan University (Rowan) to perform indoor air quality (IAQ) sampling throughout Robinson Hall on the Rowan Main Campus in Glassboro, New Jersey (Site). The sampling was requested in response to recent humidity issues that have allowed mold to proliferate within the building, and visibly grow on furniture within the Site.

The building is shut down temporarily to allow for disposal of contaminated furniture, and to allow a fine cleaning of all remaining surfaces. The purpose of this sampling is to establish a baseline for comparison before and after remediation activities have completed.

## PROJECT OBJECTIVE

The objective of this project was to collect mold spore air samples, mold surface samples, and provide recommendations as warranted, based on the laboratory analytical report.

# METHODOLOGY

The air samples were collected utilizing Air-O-Cell sampling cassettes, which are designed for collection and analysis of a wide range of airborne aerosols, including mold spores, pollen, insect parts, fibers, and other inorganic particulates.

All samples were hand delivered to EMSL Analytical, Inc. (EMSL), of Cinnaminson, New Jersey, an American Industrial Hygiene Association Environmental Microbiological Laboratory Accreditation Program (AIHA EMLAP #100194) and Industrial Hygiene Laboratory Accreditation Program (IHLAP) laboratory. All samples were submitted for a 24-hour turnaround time.

# MOLD STANDARDS

Mold growth is most likely found in areas that have sufficient moisture, temperature, and nutritive sources to promote proliferation. Nutritive sources within buildings include drywall paper backing, cellulose ceiling tiles, wallpaper, wood wall framing and trim, pipe insulation wrappings, and similar materials.

As an industry guideline, interpretation of results should be based on comparison of fungi collected from indoor air and outdoor air. Outdoor results are highly variable and can vary based on seasonal conditions and the time-of-day. Additionally, the species of fungi identified in samples collected indoors may vary from those species identified in outdoor samples. Indoor results should be comparable to the types and concentration of fungi outdoors. In some instances, indoor air concentrations of total airborne culturable fungi can exceed outdoor air concentrations, in response to outdoor environmental conditions. The presence of certain indoor-associated species may be indicative of elevated moisture and/or water-damaged building materials. Moreover, the second- edition of the American Industrial Hygiene Association (AIHA) Field Guide for the Determination of Biological Contaminants in Environmental Samples, states that dominance in indoor-air samples by species of molds that are not predominant outdoor species, indicates that molds are growing in the building and that indoor air quality is degraded.

Montrose evaluated the mold spore results using the AIHA Recognition, Evaluation, and Control of Indoor Mold, 2008. Additionally, variability in sampling conditions is taken into account during the interpretation of laboratory analytical data.

# MOLD ASSESSMENT AND MOISTURE INPSECTION ACTIVITIES

## VISUAL OBSERVATIONS

On August 23, 2023, visible suspect mold growth was observed on furniture and surfaces throughout the Site, with representative photos below:

Photo 1. Visible Suspect Mold Growth on Chair (Cloth)

Photo 2. Visible Suspect Mold Growth on Chair (Faux Leather)

Photo 3. Visible Suspect Mold Growth on Desk

Approximately 750 chairs were distributed across all three floors, and all were slated for removal, as they all appeared similarly to those represented in Photos 1 & 2. Suspect visible mold growth was identified on several tables and horizontal surfaces throughout the Site, such as the above depicted in Photo 3.

## ANALYTICAL SAMPLING AND RESULTS

On August 23, 2023, Montrose collected a total of thirty Air-O-Cell samples at the Site. Samples were collected throughout the Site, with approximately ten collected per floor, as directed by the Rowan representative. One additional sample was collected outside the building, for baseline purposes and to determine outdoor background levels of spore concentrations.

The laboratory data indicates that the indoor air contains elevated counts of Aspergillus/Penicillium of varying amounts throughout the Site, ranging from 70 spores per cubic meter (s/m3) to 17,000 s/m3, with a majority over 1,000 s/m3. Cladosporium levels were also elevated throughout the Site. Elevated amounts of both species, in exceedance of the outdoor counts of both, are of indicative of proliferating mold within the Site. Air sample laboratory analytical reports have been included in Appendix A.

## IDENTIFIED MOLD SPECIES

The spore types detailed below were analytically determined to be airborne in the samples collected within the Site:

Alternaria is one of the most common fungi worldwide. Its colonies are general found in soil,

dead organic debris, on food stuffs, textiles, and weakened plants with discolored leaves. Airborne spores are usually indicative of a humid environment when found indoors, but in most instances, spores found indoors originated from outdoor sources.

Ascospores are discharged during periods of high humidity or rain and are commonly found growing indoors on wet or damp substrates.

Aspergillus/Penicillium is commonly found in soil, plants, and rotting materials. Elevated concentrations are often indicative of water-damaged building materials. It can also be found in the dust at home, carpeting, heating and air conditioning ducts. This fungus commonly settles in human lungs and sinuses.

Basidiospores are released during periods of high humidity or rain and are commonly found in gardens or wooded areas. Mold growth associated with Basidiospores contribute to the destruction of structural building lumber. This spore is a common allergen and may contribute to symptoms associated with hay fever and asthma, and further contribute to symptoms associated with hypersentivity pneumonitis.

Bipolaris/Drechslera is often found in soil, plant debris, and as a plant pathogen on grass with a dark grey to brown coloration, and is found worldwide. For indoor growth this fungus can grow on water damaged materials, food stuffs, houseplants, and other organic materials. These molds are highly common in both indoor and outdoor environments, and most people have some level of exposure on a daily basis. May cause or exacerbate asthma, conjunctivitis, rhinitis, anaphylaxis, angioedema, hives, or pneumonitis and sinusitis in sensitive individuals.

Cercospora is often found in plants as a parasite. It is a fungus that causes purple, brown, or black spots on a variety of garden vegetables and shrubs, and is not commonly found indoors, unless interior plants are present.

Cladosporium is commonly found in soil, plants, and decaying plant materials. Concentrations may be found on many different substrates, including but not limited to: textiles, wood, and window sills. Cladosporium is a common allergen and may contribute to symptoms associated with hay fever and asthma, and further contribute to symptoms associated with hypersensitivity pneumonitis.

Curvularia is often found in plant debris, soil, facultative plant pathogens or tropical plants with a gray or brown coloration. Curvularia is a relatively common cause of allergic fungal sinusitis, specifically within immunocompromised individuals.

Epicoccum is commonly found on a variety of household materials, including wood, drywall, fabric, carpet, mattresses and painted surfaces. This species is commonly found in plant debris and soil. Presence of this mold can trigger allergic reactions and can also cause respiratory problems.

Ganoderma is often found growing on conifers and hardwoods causing white rot, root rot, and stem rot, and is considered to be an allergen. Not often found indoors, and only usually present

on rotting wood surfaces.

Myxomycetes is found usually in decaying logs, stumps, and dead leaves, particularly in forested regions. Occasionally found indoors on damp building materials such as cardboard, wallpaper, gypsum and wood, it is considered a slime-mold, and can be found world-wide. In sensitive individuals, allergic reactions may occur, including asthma, conjunctivitis, rhinitis, dermal swelling and hives.

Pithomyces is often found in dead leaves, especially leaf fodders as well as in soil and grasses with a tan to brown coloration. It is rarely found indoors unless growing on paper or houseplants.

Polythrincium is often found in dead leaves and is rarely found indoors. Spores may be seen in dust as part of the normal influx of outdoor microbial particles to indoor environments.

Rust is often found in grasses, flowers, trees, and other living plant materials. For growth this fungus requires a living host plant for growth, so it is not found on interior building components. As an allergen, it has been known to cause hay fever and asthma.

To date, there are no threshold limits or recommendations for levels of indoor mold aeroallergens. The Occupational Safety and Health Administration (OSHA), the National Institute of Occupational Safety and Health (NIOSH), as well as other occupational health related associations, have not established permissible exposure levels (PELs), recommended exposure limits (RELs), or other compliance limit values for aeroallergens. In general, spore counts and spore types are influenced by geographic location, seasonal and diurnal cycles, as well as outdoor conditions. Therefore, it is difficult for regulatory agencies and industry groups to establish permissible exposure limits for mold and fungi exposure, and as a result, there are no regulatory quantitative exposure limits for airborne mold or fungi.

# CONCLUSIONS AND RECOMMENDATIONS

Based on the August 23, 2023, visual inspection and sampling, mold is present and proliferating within Robinson Hall.

Montrose understands that Rowan has engaged AllRisk, a mold remediation contractor, to perform cleanup and disposal activities throughout Robinson Hall. Montrose recommends the following:

- All water damaged drywall and ceiling tiles to be immediately removed from the site. Dehumidifiers should be kept in place, if possible, to reduce relative humidity.

- All cardboard boxes, cork boards, crates, coverings, and furniture throughout the affected areas should be discarded, if possible. Spores have likely settled on the existing materials.

  - If disposal is not an option, those items, when possible, should be thoroughly cleaned and removed from the room.

- All partition/dividing walls (non-concrete ones), carpeting and flooring in the affected areas should be cleaned and treated with an antimicrobial coating, such as Microban, or equivalent.

- Any walls that are drywall should be examined for mold damage, and wiped down, if possible (most walls are concrete).

- All horizontal surfaces, including flooring, throughout building should be wet-wiped and vacuumed with a High Efficiency Particulate Air (HEPA) filter, to collect any spores that may have settled.

Once all areas have been cleaned by AllRisk, Montrose recommends that Rowan perform clearance air testing, to assess the efficacy of all cleaning and disposal activities, prior to the building being released for general re-occupancy.

### -o.0.o-

These results represent the conditions and concentrations present at the time of this survey. Respectfully Submitted,

MONTROSE ENVIRONMENTAL, INC.

Jessica Perrini	Julian Fernandez-Obregon

Project Health & Safety Specialist	Director, Health & Safety Division

APPENDIX A

Laboratory Analytical Reports

Microbiology Chain of Custody

EMSL Order Number (Lab Use Only):

PHONE:

FAX:

| Company: Montrose Enviromental |  |  |  |  |  |  |  | EMSL-B111 to:,wDifferentu:same If Bill to is Different no e instructions in Commen s.. Third Party Billina reauires written authorization from third oarty |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Street: 500.Horizon Drive, Suite 540 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cltv: Robbinsville \ State/Province: NJ |  |  |  |  |  |  |  | Zip/Postal Code: 08691 \ Country: USA |  |  |  |  |  |  |  |  |
| Report To ( I ): Julian Fernandez-Obregon Name |  |  |  |  |  |  |  | Telephone#: 609-890-7277 |  |  |  |  |  |  |  |  |
| Email Address: jfernandez@montrose-env.com |  |  |  |  |  |  |  | Fax#: 609-890-9116 \ Purchase Order: 1200-62 |  |  |  |  |  |  |  |  |
| Project Name/Number: Rowan-Robinson Hall 1200-62 |  |  |  |  |  |  |  | Please Provide Results: 0FAX [KjE-mail nMail |  |  |  |  |  |  |  |  |
| U.S. State Samoles Taken: NJ |  |  |  |  |  |  |  | Connecticut Samples: D Commercial D Residential |  |  |  |  |  |  |  |  |
| ' Turnaround Time (TAT) Options" - Please Check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D 3 Hour I LJ 6 Hour I 1■124 Hour |  |  | D 48 Hour I D 72 Hour I D 96 Hour |  |  |  |  |  |  |  |  |  | I 11 Week I O2Week |  |  |  |
| *Analysis completed in accordance with EMSL's Terms and Conditions located in the Analytical Price Guide. TATs are subject to methodology requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non Culturable Air Samples (Spore Traps) - Test Codes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - M001 Alr-O-Cell - M049 BioSIS - M030 Micro 5 |  | - M173 Allegro M2 - M003 Burkard - M174 MoldSnap |  | - M004 Allergenco - M043 Cyclex - M176 Relle Smart |  |  |  |  | - M032 A!le enco-D - M002 Cyclex-d - M130 Via-Cell |  |  |  |  | - M172 Versa Trap |  |  |
| Other Microbioloav Test Codes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - M041 Fungal Direct Examination - MOOS Viable Fungi 1D and Count .• MOOG Viable Fungi ID and Count (Speciation) M007 Culturable Fungi - MOOS Culturable Fungi (Speciation) - M009 Gram Stain Culturable Bacteria - M010 Bacterial Count and ID - 3 Most Prominent ·,• M011-Bacterial Ccrunt--anctlD'= S-Mosr -c - Prominent - M013 Sewage Contamination in Buildings |  |  | - M014 Endotoxin Analysis - M015 Heterotrophic Plate Count - M180 Real Time Q-PCR-ERMI 36 - Panel - M018 Total Colifom, (Membrane Filtration) - M020 Fecal Streptococcus (Membrane Filtration) - M210-215 Legionef/-a Detection - M026 Recreational Water Screen - M027 Mycotoxin Analysis |  |  |  |  |  |  |  | - M029 Enterococci - M019 Fecal Coli - M133 MRSA An is £? - M028 Cryptoco(!6l1s nelifDrmans Detection :,:: ::o - M120 HistoplafNJj ca ffili!i.!1! Detection w ====:g - M033-3906}Ler. n-Te -- - - - M044 Group All gen o n, (Cat, Dog, Cockroaci.=ousii:ilites) - Other See Anaiy°tical ice Guide |  |  |  |  |  |
| . ,, Preservation Method (Water): .o |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name of Sampler; Jessica Perrini |  |  |  |  |  |  | ' SiAnature ofSampler: 1h11 |  |  |  |  |  |  |  |  |  |
| Sample# | Sample Location |  |  |  | Sample Tvoe |  |  |  | Test Code | V,rlu., - '--""' me/Area |  |  |  |  | Date/Time Collected |  |
| Example:A1 | Kitchen |  |  |  | Air |  |  |  | M001 | 75L |  |  |  |  | 1/1/12 4:00 PM |  |
| ·Rt>(J- 0 I | · ,A |  |  |  | Air |  |  |  | M001 | 150L |  |  |  |  | 8/23/2023 |  |
| oLa.:.t'l, | o II |  |  |  | Air |  |  |  | M001 |  |  |  |  |  |  |  |
| D I> u· - t'l "t- | ';? ?t\ |  |  |  | Air |  |  |  | M001 |  |  |  |  |  |  |  |
| 001.J- O<.f | "'i>- °'" eLt |  |  |  | Air |  |  |  | M001 |  |  |  |  |  |  |  |
| .o '- 1i- o - | o- 3&" ,.._ |  |  |  | Air |  |  |  | MD01 |  |  |  |  |  |  |  |
| Q o- Ob | 'i? fVt\... - ..! 3 |  |  |  | Air |  |  |  | M001 • |  |  |  |  |  |  |  |
| Q fl. u - e .., | li-""" t'- \ |  |  |  | Air |  |  |  | M001 |  |  |  | I |  |  | I |
| Qou- ov | "Q> I\.. -- 64 |  |  |  | Air |  |  |  | M001 | \ |  |  | I |  | \ | I |
| DI.I- 0 ct | -e,"" t-lll.\\wr..,. ,. 2 |  |  |  | Air |  |  |  | M001 |  |  |  | ' I |  | \I/ |  |
| CliontSampl # (s): I - I |  |  |  |  |  |  |  | Total# of Samples: .,OKl |  |  |  |  |  |  |  |  |
| ' A Relinquished, (Client): - - |  |  |  |  |  | Date: V/l7/2"t |  |  |  |  |  | Time: |  |  |  |  |
| Received (Clientla_ / |  |  |  |  |  | Oate:<J/J-3/ |  |  |  |  |  | Time: / '3on . |  |  |  |  |
| Comments: .,. / - / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Page 1 of _J,_ pages

■

Page 1 Of	2

Microbiology Chain of Custody

EMSL Order Number (Lab Use Only):

PHONE:

FAX:

Additional Pages of the Chain of Custody are only necessary if needed for additional sample information

| Sample# |  | Sample Location | Sample Type | Test Code | Volume/Area |  | Date/Time |  | Collected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R. u-10 |  | t,\la\\uJ ti Lt ft,\ ] J\ | Air | M001 | 150L |  | 8/23/2023 |  |  |
| "'0\.A-lt |  | I M. 2-0; | Air | M001 | I |  | I |  |  |
| e» e.1.A - \? |  | Mo) \t\ | Air | M001 | \ |  |  |  |  |
| 0 | \A-\ | 'M. d0d. | Air | M001 |  |  | -= |  |  |
| Q C>. - IL\ |  | 'Hh\.\ \11 lHA O I\ 2f2 i, 2J1 | Air | M001 |  |  | ...... | --C") |  |
| P.\A-- IS- |  | \\.-'\ I' (\ | Air | M001 |  |  | cE;-, | z J> | ;;o rn |
| '2..e ., |  | o k--\ e)o/.,, | Air | M001 |  |  | Nw | r--,-c-...> Z en- |  |
| Q | .. \) | -a, \'\-\ d O .S- | Air | M001 |  |  | 1} | az ' ;;, !'--i |  |
| «Q. f...t- I y |  | re .:;)0)5 | Air | M001 |  |  | . | i-2: |  |
| C>-0 IA. -1, C, |  | e...Y\.\ ;JO l.f | Air | M001 |  |  | 0 |  |  |
| a o kl::-2.._b_ (,?i2.l..1 -i:-r- |  | -R""... J h | Air | M001 |  |  |  |  |  |
|  |  |  | Air | M001 |  | - | -- | - - - |  |
| f.r2.t.-t· 27 |  | .p M IOI ls't - | Air | M001 |  |  |  |  |  |
| fl, Lt ... 2 'l |  | t'½ } | Air | M001 |  |  |  |  |  |
| Q (LlJ - 2tJ |  | -RVL-i-JCJ2. | Air | M001 |  |  |  |  |  |
| 0<2.\A 'l..\ |  | p_""1. - ' I r- | Air | M001 |  |  |  |  |  |
| R.2U- 2b |  | ,l. V'-1 l () S' | Air | M001 |  |  |  |  |  |
| _Y2.. lJ.. 21 |  | (( '(\.,\ -/QJ i? | Air | M001 |  |  |  |  |  |
| () t2.u- k |  | tfc.1.\\uih l .. () (\,\ \ \ 9(° | Air | M001 |  |  |  |  |  |
| ' 'O l)· 4 |  | \\'1..\\ h..Lt' "\'\I\ Jl\\l\ | Air | M001 |  |  |  |  |  |
| O V...Jo |  | I {t'\U-Jcl.tr.lt \rr r, | Air | M001 |  |  |  |  |  |
|  |  | () | Air | M001 |  |  | \ |  |  |
|  |  |  | Air | M001 | \ | I | \ | I |  |
|  |  |  | Air | M001 | 'v |  | \/- |  |  |
| ...Comments/Special Instructions: |  |  |  |  |  |  |  |  |  |

Page   2:_ of d-  pages

Page 2 Of	2

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0001 RRU-01 150 |  |  | 372313210-0002 RRU-02 150 |  |  | 372313210-0003 RRU-03 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 350 |  |  | Rm 311 |  |  | Rm 302 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | 9 | 200 | 6.7 | - | - | - | 3 | 70 | 0.6 |
| Aspergillus/Penicillium | 6 | 100 | 3.4 | 1 | 20 | 6.3 | 488 | 10600 | 92.2 |
| Basidiospores | 57 | 1200 | 40.4 | 12 | 260 | 81.3 | 5 | 100 | 0.9 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 62 | 1400 | 47.1 | 2 | 40 | 12.5 | 33 | 720 | 6.3 |
| Curvularia | 1 | 20 | 0.7 | - | - | - | 1* | 7* | 0.1 |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | 2 | 40 | 1.3 | - | - | - | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | - | - | - | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | 2* | 10* | 0.3 | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 139 | 2970 | 100 | 15 | 320 | 100 | 530 | 11497 | 100 |
| Hyphal Fragment | - | - | - | 2* | 10* | - | 1 | 20 | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 1 | - | - | 2 | - | - | 1 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 2 | - | - | 1 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 1 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0004 RRU-04 150 |  |  | 372313210-0005 RRU-05 150 |  |  | 372313210-0006 RRU-06 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 306 |  |  | Rm 306A |  |  | Rm 303 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | 1 | 20 | 0.1 | - | - | - |
| Ascospores | 1 | 20 | 2.9 | - | - | - | 4 | 90 | 2.9 |
| Aspergillus/Penicillium | 8 | 200 | 28.6 | 781 | 17000 | 97.1 | 22 | 480 | 15.5 |
| Basidiospores | 18 | 390 | 55.7 | 1 | 20 | 0.1 | 62 | 1400 | 45.3 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 3 | 70 | 10 | 20 | 440 | 2.5 | 49 | 1100 | 35.6 |
| Curvularia | - | - | - | - | - | - | 1 | 20 | 0.6 |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | 1 | 20 | 2.9 | - | - | - | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | 1 | 20 | 0.1 | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 31 | 700 | 100 | 804 | 17500 | 100 | 138 | 3090 | 100 |
| Hyphal Fragment | - | - | - | 1 | 20 | - | - | - | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | 1* | 7* | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 2 | - | - | 1 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 3 | - | - | 2 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 2 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0007 RRU-07 150 |  |  | 372313210-0008 RRU-08 150 |  |  | 372313210-0009 RRU-09 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm G1 |  |  | Rm 304 |  |  | Rm Hallway Rm 323 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | - | - | - | - | - | - | 1 | 20 | 0.3 |
| Aspergillus/Penicillium | 98 | 2100 | 65 | 341 | 7440 | 67.6 | 314 | 6850 | 96.8 |
| Basidiospores | 12 | 260 | 8 | 3 | 70 | 0.6 | 4 | 90 | 1.3 |
| Bipolaris++ | - | - | - | - | - | - | 1 | 20 | 0.3 |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 39 | 850 | 26.3 | 159 | 3470 | 31.5 | 5 | 100 | 1.4 |
| Curvularia | - | - | - | - | - | - | - | - | - |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | 1 | 20 | 0.6 | - | - | - | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | 1 | 20 | 0.2 | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | 1* | 7* | 0.1 | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 150 | 3230 | 100 | 505 | 11007 | 100 | 325 | 7080 | 100 |
| Hyphal Fragment | 2 | 40 | - | 2 | 40 | - | - | - | - |
| Insect Fragment | 1 | 20 | - | - | - | - | - | - | - |
| Pollen | - | - | - | 1* | 7* | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 2 | - | - | 2 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 2 | - | - | 2 | - | - | 2 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 3 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0010 RRU-10 150 |  |  | 372313210-0011 RRU-11 150 |  |  | 372313210-0012 RRU-12 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hallway Rm 311 |  |  | Rm 207 |  |  | Rm 201A |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | 2 | 40 | 0.7 | 1 | 20 | 1.5 | - | - | - |
| Aspergillus/Penicillium | 233 | 5080 | 90.7 | 30 | 660 | 50.5 | 45 | 980 | 62.9 |
| Basidiospores | 20 | 440 | 7.9 | 21 | 460 | 35.2 | 17 | 370 | 23.8 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 2 | 40 | 0.7 | 5 | 100 | 7.7 | 7 | 200 | 12.8 |
| Curvularia | - | - | - | 1* | 7* | 0.5 | - | - | - |
| Epicoccum | - | - | - | 1 | 20 | 1.5 | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | 2 | 40 | 3.1 | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | - | - | - | 1* | 7* | 0.4 |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 257 | 5600 | 100 | 61 | 1307 | 100 | 70 | 1557 | 100 |
| Hyphal Fragment | - | - | - | - | - | - | - | - | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 1 | - | - | 1 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 1 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 4 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0013 RRU-13 150 |  |  | 372313210-0014 RRU-14 150 |  |  | 372313210-0015 RRU-15 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 202 |  |  | Hallway Rm 212 & 213 |  |  | Rm 230 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | - | - | - | 1 | 20 | 1.3 | - | - | - |
| Aspergillus/Penicillium | 127 | 2770 | 91.4 | 45 | 980 | 65.9 | 45 | 980 | 57.3 |
| Basidiospores | 1 | 20 | 0.7 | 12 | 260 | 17.5 | 28 | 610 | 35.7 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 10 | 220 | 7.3 | 10 | 220 | 14.8 | 5 | 100 | 5.8 |
| Curvularia | - | - | - | - | - | - | - | - | - |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | - | - | - | 1 | 20 | 1.2 |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | 1 | 20 | 0.7 | 1* | 7* | 0.5 | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 139 | 3030 | 100 | 69 | 1487 | 100 | 79 | 1710 | 100 |
| Hyphal Fragment | - | - | - | - | - | - | 1 | 20 | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 1 | - | - | 2 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 1 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 5 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0016 RRU-16 150 |  |  | 372313210-0017 RRU-17 150 |  |  | 372313210-0018 RRU-18 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 206 |  |  | Rm 205 |  |  | Rm 201B |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | - | - | - | 2 | 40 | 1 | - | - | - |
| Aspergillus/Penicillium | 139 | 3030 | 25.3 | 125 | 2730 | 68.1 | 101 | 2200 | 76.4 |
| Basidiospores | 19 | 420 | 3.5 | 44 | 960 | 23.9 | 10 | 220 | 7.6 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 392 | 8550 | 71.3 | 12 | 260 | 6.5 | 20 | 440 | 15.3 |
| Curvularia | - | - | - | - | - | - | 1 | 20 | 0.7 |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | 1 | 20 | 0.5 | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | - | - | - | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 550 | 12000 | 100 | 184 | 4010 | 100 | 132 | 2880 | 100 |
| Hyphal Fragment | - | - | - | - | - | - | 1 | 20 | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 1 | - | - | 2 | - | - | 2 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 1 | - | - | 2 | - |

372313210-0016 - Aspergillus conidiophores present in sample.

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 6 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0019 RRU-19 150 |  |  | 372313210-0020 RRU-20 150 |  |  | 372313210-0021 RRU-21 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 204 |  |  | Hallway Rm 216 |  |  | Rm 100 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | 1* | 7* | 0.5 |
| Ascospores | - | - | - | - | - | - | - | - | - |
| Aspergillus/Penicillium | 139 | 3030 | 76.3 | 132 | 2880 | 93.2 | 63 | 1400 | 96.8 |
| Basidiospores | 2 | 40 | 1 | 4 | 90 | 2.9 | 1 | 20 | 1.4 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 41 | 900 | 22.7 | 5 | 100 | 3.2 | 1 | 20 | 1.4 |
| Curvularia | - | - | - | 1 | 20 | 0.6 | - | - | - |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | - | - | - | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | - | - | - | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 182 | 3970 | 100 | 142 | 3090 | 100 | 66 | 1447 | 100 |
| Hyphal Fragment | 2 | 40 | - | 1 | 20 | - | - | - | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 2 | - | - | 1 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 2 | - | - | 2 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 7 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0022 RRU-22 150 |  |  | 372313210-0023 RRU-23 150 |  |  | 372313210-0024 RRU-24 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RM 101A |  |  | RM 105 |  |  | Rm 102 |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | 1 | 20 | 5.7 | 4 | 90 | 5.4 | - | - | - |
| Aspergillus/Penicillium | 3 | 70 | 20 | 8 | 200 | 12 | 240 | 5240 | 91.9 |
| Basidiospores | 10 | 220 | 62.9 | 58 | 1300 | 78 | 19 | 420 | 7.4 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 2 | 40 | 11.4 | 3 | 70 | 4.2 | 2 | 40 | 0.7 |
| Curvularia | - | - | - | - | - | - | - | - | - |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | 1* | 7* | 0.4 | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | - | - | - |
| Pithomyces++ | - | - | - | - | - | - | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 16 | 350 | 100 | 74 | 1667 | 100 | 261 | 5700 | 100 |
| Hyphal Fragment | - | - | - | - | - | - | - | - | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 1 | - | - | 1 | - | - | 2 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 1 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 8 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0025 RRU-25 150 |  |  | 372313210-0026 RRU-26 150 |  |  | 372313210-0027 RRU-27 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rm 115 |  |  | Rm 103 |  |  | Rm 101B |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | - | - | - |
| Ascospores | 3 | 70 | 4.4 | 1 | 20 | 0.9 | - | - | - |
| Aspergillus/Penicillium | 20 | 440 | 27.8 | 44 | 960 | 41.2 | 6 | 100 | 16.7 |
| Basidiospores | 45 | 980 | 62 | 16 | 350 | 15 | 12 | 260 | 43.3 |
| Bipolaris++ | - | - | - | - | - | - | - | - | - |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 4 | 90 | 5.7 | 46 | 1000 | 42.9 | 10 | 220 | 36.7 |
| Curvularia | - | - | - | - | - | - | - | - | - |
| Epicoccum | - | - | - | - | - | - | - | - | - |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | - | - | - | - | - | - |
| Myxomycetes++ | - | - | - | - | - | - | 1 | 20 | 3.3 |
| Pithomyces++ | - | - | - | - | - | - | - | - | - |
| Rust | - | - | - | - | - | - | - | - | - |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | - | - | - |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | - | - | - |
| Polythrincium | - | - | - | - | - | - | - | - | - |
| Total Fungi | 72 | 1580 | 100 | 107 | 2330 | 100 | 29 | 600 | 100 |
| Hyphal Fragment | - | - | - | 1* | 7* | - | - | - | - |
| Insect Fragment | - | - | - | - | - | - | - | - | - |
| Pollen | - | - | - | - | - | - | - | - | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 1 | - | - | 2 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 2 | - | - | 1 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 9 of 10

| Lab Sample Number: Client Sample ID: Volume (L): Sample Location: | 372313210-0028 RRU-28 150 |  |  | 372313210-0029 RRU-29 150 |  |  | 372313210-0030 RRU-30 150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hallway Rm 118 |  |  | Hallway Rm 101A |  |  | Outside Bldg |  |  |
| Spore Types | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total | Raw Count | Count/m³ | % of Total |
| Alternaria (Ulocladium) | - | - | - | - | - | - | 4 | 90 | 0.8 |
| Ascospores | - | - | - | - | - | - | 30 | 660 | 6 |
| Aspergillus/Penicillium | 38 | 830 | 92.2 | 27 | 590 | 44.5 | 13 | 280 | 2.6 |
| Basidiospores | - | - | - | 20 | 440 | 33.2 | 381 | 8310 | 76.2 |
| Bipolaris++ | - | - | - | - | - | - | 1 | 20 | 0.2 |
| Chaetomium++ | - | - | - | - | - | - | - | - | - |
| Cladosporium | 3 | 70 | 7.8 | 10 | 220 | 16.6 | 40 | 870 | 8 |
| Curvularia | - | - | - | - | - | - | 1 | 20 | 0.2 |
| Epicoccum | - | - | - | 1* | 7* | 0.5 | 2 | 40 | 0.4 |
| Fusarium++ | - | - | - | - | - | - | - | - | - |
| Ganoderma | - | - | - | - | - | - | 19 | 420 | 3.8 |
| Myxomycetes++ | - | - | - | - | - | - | 2 | 40 | 0.4 |
| Pithomyces++ | - | - | - | 3 | 70 | 5.3 | 2 | 40 | 0.4 |
| Rust | - | - | - | - | - | - | 3* | 20* | 0.2 |
| Scopulariopsis/Microascus | - | - | - | - | - | - | - | - | - |
| Stachybotrys/Memnoniella | - | - | - | - | - | - | - | - | - |
| Unidentifiable Spores | - | - | - | - | - | - | 2 | 40 | 0.4 |
| Zygomycetes | - | - | - | - | - | - | - | - | - |
| Arthrinium | - | - | - | - | - | - | - | - | - |
| Cercospora++ | - | - | - | - | - | - | 1 | 20 | 0.2 |
| Polythrincium | - | - | - | - | - | - | 2 | 40 | 0.4 |
| Total Fungi | 41 | 900 | 100 | 61 | 1327 | 100 | 503 | 10910 | 100 |
| Hyphal Fragment | 1 | 20 | - | 3 | 70 | - | 1 | 20 | - |
| Insect Fragment | - | - | - | - | - | - | 1 | 20 | - |
| Pollen | - | - | - | 1 | 20 | - | 3 | 70 | - |
| Analyt. Sensitivity 600x | - | 22 | - | - | 22 | - | - | 22 | - |
| Analyt. Sensitivity 300x | - | 7* | - | - | 7* | - | - | 7* | - |
| Skin Fragments (1-4) | - | 2 | - | - | 1 | - | - | 1 | - |
| Fibrous Particulate (1-4) | - | 1 | - | - | 1 | - | - | 1 | - |
| Background (1-5) | - | 1 | - | - | 4 | - | - | 3 | - |

++ Includes other spores with similar morphology; see EMSL's fungal glossary for each specific category.

No discernable field blank was submitted with this group of samples.

Vincent Iuzzolino, M.S., Laboratory Manager or other Approved Signatory

Skin Fragment and Fibrous Particulate ratings are based on the percent of non-fungal material they represent: 1 (1-25%), 2 (26-50%), 3 (51-75%), or 4 (76-100%). Background ratings are based on the total area covered by non-fungal particles: 1 (1-25%), 2 (26-50%), 3 (51-75%), 4 (76-99%), or 5 (100%; overloaded, prohibiting accurate detection and quantification). High levels of background will obscure spores and other particulates, leading to underestimation. Present = Spores detected on overloaded samples. Results are not blank corrected unless otherwise noted. The detection limit is equal to one fungal spore, structure, pollen, fiber particle or insect fragment. "*" Denotes particles found at 300X. "-" Denotes not detected. Due to method stopping rules, raw counts in excess of 100 are extrapolated based on the percentage analyzed. EMSL maintains liability limited to cost of analysis.

Interpretation and use of test results are the responsibility of the client. This report relates only to the samples reported above, and may not be reproduced, except in full, without written approval by EMSL. EMSL bears no responsibility for sample collection activities or analytical method limitations. The report reflects the samples as received. When the information supplied by the customer can affect the validity of the result, it will be noted on the report.

Samples analyzed by EMSL Analytical, Inc. Cinnaminson, NJ AIHA LAP, LLC-EMLAP Accredited #100194

Initial report from: 08/24/2023 03:49 PM

For information on the fungi listed in this report, please visit the Resources section at www.emsl.com

MIC_M001_0002_0002 Printed: 08/24/2023 03:49 PM	Page 10 of 10