| TOMS RIVER TOWNSHIP FIRE DEPARTMENT Standard Operating Guidelines |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CHAPTER: 5 | SECTION: 03 |  | # OF PAGES: 3 |  |  |
| SUBJECT: THERMAL IMAGING CAMERA |  |  |  |  |  |
| EFFECTIVE DATE: 9/7/2016 |  | BY THE ORDER OF: Toms River Bds of Fire Commissioners Dist. 1 & 2, individually; and Toms River Joint Fire Officers Assn. |  | SUPERSEDES ORDER #: 5-03, eff. 1-1-2005 |  |

Purpose:	This procedure is designed to establish guidelines to facilitate the most effective method for deploying the thermal image camera in a way that provides the most protection for fire department personnel, and to provide a reference document to be used for training of personnel in the uses, deployment, limitations, operation, care and maintenance of the Thermal Image Camera (TIC).

Scope:	This guideline will apply to the use of all thermal imaging cameras assigned to Toms River Fire Companies.

Safety: 	Use of a TIC during emergency operations will provide the means for locating victims and fire in a more expeditious manner. This will greatly enhance the safety of personnel by limiting time spent in the hazardous area searching for victims and the seat of fire. It must be remembered that a TIC is a tool to be utilized by firefighters and will be used to augment sound strategy, tactics and time tested interior structural firefighting and rescue procedures.

- Thermal Image Camera Uses

  - Provides safer navigation in a space where there is diminished visibility due to smoke.

  - Allow personnel to "see" in a diminished visibility environment, which is a very useful addition to traditional search and rescue techniques. Utilizing a TIC can substantially reduce the time necessary for completing a primary search.

  - Enables suppression crews to execute a faster, more effective interior attack. The shortest route to the fire, holes in the floor and obstacles in the structure can be determined and located efficiently.

  - When used from the exterior, allows personnel outside the affected structure (including the Incident Commander and outside vent crews) to better observe the seat of the fire, by indicating areas of higher heat.

  - Reduces fatigue of interior crews because efficiency in performing searches and suppression is increased.

  - Allows Rapid Intervention Teams to quickly and efficiently locate downed firefighters.

  - May be used to determine fluid level within a container, which may be useful during an incident involving a hazardous material.

  - May be used as a search tool to locate lost persons in open wilderness areas.

- Background Information

- Thermal energy is not visible to the human eye, but the firefighter can feel the heat present. The TIC allows a thermal heat view of one's surroundings.

- When viewing a room using the TIC, hot things appear white, hotter objects appear brighter white, and colder items appear black to gray. The whiter the representation displayed, the more heat present in the object.

- Limitations

  - The TIC allows a two dimensional view of a smoke filled environment. Depth perception is limited.

  - Thermal energy does not travel directly through walls. If fire is present inside a wall, the camera will only be able to "see" it if the fire has increased the temperature of the wall itself.

- A human being will not provide sufficient thermal energy to penetrate most standard construction materials or solid items such as furniture. Therefore, it is reinforced that while conducting a search, rescuers must look under/ around beds, sofas and other objects and in closets where victims may have hidden to escape fire.

- Water, plastic and glass are all effective barriers for the TIC and may cause a reflective image.

- The TIC must be used with the understanding that it is only a mechanical device and it can fail. Firefighters must plan for this possibility by carrying flashlights, maintaining contact with the wall, a hose line, employing a tag line or other routine methods for remaining oriented to location and the position of exits in a diminished visibility environment. Crews should continue to employ standard firefighting practices.

- The image displayed by the TIC may decrease in quality as soot builds up on the lens and screen while operating on the fire ground. A soft cloth or a gloved hand should be used to clean the lens and screen periodically while operating the camera.

- If the picture displayed on the screen suddenly becomes distorted or dark: a. Check to insure the carrying strap is not in front of the lens

- The Thermal Imaging Camera has not been determined to be intrinsically safe as an ignition source. This device is not to be used in a potentially explosive atmosphere.

- Incident Operation of the TIC

- Personnel should become familiar with the location of the TIC on the apparatus.

- The TIC shall go in with the OIC/Team Leader of the attack crew on all calls, where the Team Leader/TIC operator should be directly behind the nozzle operator or should lead a team where hoselines are not required to be deployed. Search and suppression activities should occur in compliance with their respective S.O.G.s and standard firefighting practices should be observed with the TIC acting as an "extension of the hand."

- The Camera has the potential to inspire overconfidence because it allows firefighters to "see" in an environment that in reality has zero visibility. Personnel must understand that the camera could fail and an escape route must be easily located, either by staying oriented and/or by following a hose line or rope tag line to safety.

- It is important for firefighters to allow sufficient time to exit a hazardous atmosphere when the battery status indicator shows that the battery power is getting low.

- Inspection Procedures

- The camera should be checked as part of the (weekly) equipment check of the apparatus to which it is assigned.

- The camera should be inspected for cleanliness. If any part of the camera is dirty a clean rag dampened with soapy water should be used to clean the camera. No harsh detergents or solvents should be used.

- The camera should be turned on and checked for proper operation and then turned off.

- Problems with the unit should be reported to The Fire Chief or any line officer.

- Maintenance

- Batteries should be rotated periodically.

- After the camera is used on an incident it should be thoroughly cleaned and dried before it is returned to the charger.

- Training

- TIC use should be utilized as much as possible during company drills to ensure all personnel understand it’s applications and limitations, and how each model functions.