| TOMS RIVER TOWNSHIP FIRE DEPARTMENT Bureau of Fire Prevention |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CHAPTER: 7 | SECTION: 11 |  | # OF PAGES: 5 |  |  |
| SUBJECT: Fire Investigations |  |  |  |  |  |
| EFFECTIVE DATE: August 9, 2023 |  | BY THE ORDER OF: Toms River Bds of Fire Commissioners Dist. 1 & 2 |  | SUPERSEDES ORDER #: |  |

Purpose: The purpose of this policy is to establish criteria to be followed by all Bureau of Fire Prevention Fire Inspectors when conducting fire investigations.

Scope: The Fire Investigation Standard Operating Guidelines (SOG) applies to all personnel responsible for conducting fire investigations for the Toms River Bureau of Fire Prevention and to establish a comprehensive framework for conducting systematic and thorough investigations of fire incidents within the jurisdiction of the Toms River Bureau of Fire Prevention.

Procedure: The Fire Official or his / her designee is responsible for determining the area/point of origin and cause of a fire. Under certain circumstances the IC will need additional assistance. The Toms River Bureau of Fire Prevention Office shall be called to the following, but not limited to, incidents:

Examples:

- At the discretion of the Fire Official.

- At the request of the Incident Commander.

- Any structural fire or explosion resulting in the damage of the structure or contents as the result of burning, smoke, heat, water, or other fire control operations.

- Any fire or explosion, regardless of the severity, which results in the injury or death to a civilian or firefighter.

- Any other fire or explosion that is of a suspicious nature.

- Any attempted incendiary act involving a structure, motor vehicle, or real property.

- A fire or explosion that is believed to be incendiary in nature

- A fire or explosion in which the cause is unknown by the Incident Commander.

- A fire in which a suspected arsonist is identified by witnesses or victims.

- Any unique or complicated situation involving a suspicious or noteworthy fire.

When the Fire Prevention Office is called, the fire company should fill out their local incident report in all areas except those relating to the cause of the fire, unless the investigator provides instructions on what to report. Where an investigation is continuing, the fire company should write, “Under investigation by The Toms River Bureau of Fire Prevention Office.”

Fire Investigation Guidelines

Upon arrival at a fire scene, the investigator will immediately confer with the incident commander to obtain information regarding the situation. Prior to commencing scene examination, the investigator shall take the necessary precautions to prevent entry into the scene by unauthorized persons or non-essential fire department personnel. Entry/overhaul of any structure or vehicle will be closely coordinated between Command and the Fire Investigator.

The following may be utilized to secure a scene:

- Fire line or hazardous area tape

- Police officers

- Fire department personnel

- Other means as circumstances dictate

Pre-Examination Interviews

The investigator should conduct brief interviews with first arriving fire crew to obtain information relative to conditions encountered, forced entry mode, or any observations that will assist the fire investigator.

The fire investigator should also conduct brief interviews of witnesses, bystanders, or other persons who may provide insight as to the cause of the incident. Obtain the following minimum information:

- Name

- Phone number (cell or emergency contact)

- Address

- Date of birth

- Witness statement

Police Department Investigative Assistance

In the event that the fire investigator determines the cause to be incendiary or attempted arson, the incident commander and/or the Fire Inspector shall request the Toms River Police Department (TRPD) and the Ocean County Prosecutors Office respond to the scene. Notification to the Toms River Fire Bureau Chief shall be made immediately as well.

- Notifications shall be made to the Ocean County Prosecutors Office for all incidents as stated in the most current OCPO Arson Response Policy (LED – 2019 – 005). This policy shall be attached as reference and updated as directed by the Ocean County Prosecutors Office.

Upon the arrival of the police detective, the fire investigator shall brief the police detective as to the scene and initial findings. For the purpose of the initial investigation, the police investigator is a member of the fire investigation team.

When working in conjunction with detectives, interviewing of witnesses shall be done in with representatives from both agencies. All interviews and statements are to be accurately documented.

Safety Equipment

Prior to entering an emergency scene, the investigator shall wear safety equipment as the scene dictates (i.e., hard hat, boots, dust mask, bunker gear or Tyvek suit, etc.). Investigators shall not enter structures until atmospheric conditions are tenable.

- Investigator will coordinate with fire company officers or IC, as situation dictates, for equipment needs.

Photographs

Photographs taken at incident scenes will become part of the investigative report. A photo log may be provided for photographs taken in conjunction with investigations. Photo documentation of incidents should be thorough and complete. All areas of the incident including interior, exterior, evidence discovered, and extent of damage, etc. shall be photographed. When working in conjunction with the TRPD, OCPO or other agencies, photographs shall be copied to all investigators assigned to the investigation. When possible, it is preferred that one agency photograph the investigation and scene.

Evidence Collection and Preservation

When working in conjunction with the TRPD, OCPO or other agencies, the law enforcement detective/investigator assigned to the investigation shall perform all evidence collection, documentation of chain of custody and communication with any testing labs or other agencies relative to the evidence.

Scene Examination

When investigating fires/explosions the investigator shall use the scientific methodology to develop a hypothesis, test the hypothesis, and draw a conclusion.

1. Structure Fires

- Survey and photograph the exterior of the structure for evidence of external sources of ignition and physical evidence. Photograph/note the condition/position of doors, windows, and locking devices.

- Examine and document all of the utility services (gas, electric, telephone, cable for fire cause and service status). A technician from the utility company may be utilized for this examination.

- Enter the structure when atmospheric conditions permit to determine area of origin. Document from least to most damage.

- Once the area of origin has been determined, examine this area thoroughly for fire cause. The cause determination should be documented with photographs and sketches as necessary.

- Evidence of incendiary fires should be properly collected, documented and submitted for analysis.

- After completing the scene examination, the building should be turned over to a responsible party.

2. Fatal fires

- Secures the scene immediately. Allow for only essential fire operations.

- Request a response from TRPD, OCPO or other agencies as needed.

- If the victim has been moved, i.e. resuscitation, immediately interview the firefighters who first discovered and/or moved the victim. Obtain the firefighter’s written statement as soon as possible.

- The fire investigation shall be conducted simultaneously and in cooperation with the TRPD, OCPO or other agencies.

- Law enforcement (OCPO) shall be the lead on the death investigation of all fatal fires.

3. Vehicle Fires

- Examine and document exterior of vehicle for exterior sources of ignition, body damage, tires, and physical evidence.

- Examine and document engine compartment for mechanical/electrical causes or engine condition.

- Examine and document passenger compartment for fire cause or physical evidence.

- Document license plate and VIN.

- Examine and document trunk compartment

- Photograph or sketch vehicle location showing relative location on property and extent of damage.

4. Miscellaneous Fires

Examination and documentation of these scenes should follow guidelines similar to those for structure and vehicle fires.

- Complete documentation of the scene as referenced above.

5. Explosives Incidents

For the safety of all personnel the investigation of incidents involving explosive devices shall be conducted under the direction of the TRPD, OCPO or other agencies as needed.

- Examination and documentation of accidental explosions such as dust, over pressure, gas, etc. should follow guidelines similar to those for structure and vehicle fires referenced above.

Report/Case Preparation

For every scene examination a report shall be prepared and saved in the Toms River Bureau of Fire Prevention records system by the inspector who investigated the incident. The report shall include all scene sketches, photo log, and narrative including all facts of the case and an opinion/summary as to the origin and cause of the fire. All photographs taken by the investigator of the scene/incident will be filed with the final report.

- All reports shall be completed within 2 weeks of the fire unless extenuating circumstances prevent completion.

- All reports shall be entered in to the Spillman CAD system after final review and approval.

Atmospheric Conditions

Fire scene atmospheres may contain ignitable gas, vapors, and liquids, in addition to the by-products of combustion. The atmosphere shall be tested prior to conducting an investigation. The following conditions shall be within the limits listed below:

- Oxygen (O2): >19.5% or <23.5%

- Carbon Monoxide (CO): < 30 PPM

- Lower Explosive Limit (LEL): <10%

- Hydrogen Cyanide (HCN): < 10 PPM

FIRE SCENE CONTROL (SECURITY)

If during the course of a fire investigation the fire investigator determines that there has been a crime committed, it shall be his/her responsibility to request the TRPD, OCPO or other agencies to respond to the scene. The investigator will brief the police representative on the circumstances surrounding the cause of the fire and the classification of the scene as a crime scene. At that time, the TRPD, OCPO or other agencies will be given the responsibility of providing scene security.

Conflict Resolution

Should a problem develop in any phase of the fire investigation and a solution to the problem cannot be reached among the investigators and their supervisors, the problem will then be forwarded to the respective department heads for further review and discussion.

Building Code Official Notification

A request to the Toms River Township Building Official may be created for any reason deemed necessary by the fire investigator. All fires that result in structural damage shall be reported to the Building Department prior to all fire department units clearing the incident. The fire inspector shall document the posting of all structures deemed unsafe by the Toms River Township Building Official.

- Notification to the building code official is not limited to only structural damage. The loss or disconnection of a utility may cause the need for Toms River Township Building Official to respond as well.

Consent to Search

If the Fire Prevention Office or fire investigator finds need to re-enter a scene, after all fire personnel have cleared the scene or property has been turned over to the property owner, a consent to search form must be completed.