MUNICIPAL CERTIFICATE OF CODE COMPLIANCE INSPECTION

3 MYRTLE AVENUE

BLOCK 142 / LOT 13

APRIL 23, 2024

Insp. K. Capobianco

I conducted an inspection with the following observations.  This inspection is considered “FAILED”.  All deficiencies must be abated for the MCC to be released.     A lead inspection is ordered for the entire interior of the structure due to poor condition of paint in several areas (peeling and cracking and bubbling).  This house is built prior to 1978.

Property lot –

- Still garbage and debris (rotted grill, screens, storm window, a/c unit) strewn around property.

- The grass area between the sidewalk and curb is destroyed and has been for a long time.  This area must be returned to how it was.  Many violation letters have been sent and this has not been addressed.

Structure –

Exterior:

- Repair and paint side steps to porch (east side) on Washington. Contact HPC for approval.

- Repair/paint side porch steps (west side). Safety Item. HPC.

- Repair/paint front porch steps. HPC.

- Repair/paint railings/landing (west side). Safety Item. HPC.

- Repair/paint front porch railings. Safety Item. HPC.

- Repair/paint front porch decking. HPC.

- Repair/replace any missing or damaged screens around entire house.

- Front door (main entrance) has no auto closer and the door does not latch due to it being kicked in at some point.  Safety Item.  This must be repaired.

- Inspect/repair/paint any trim areas that need it. HPC.

Interior:

Basement:

- Hot water heaters appear to have been installed without permits and do not have grounding straps. Safety Item. Contact building department to apply for permits and inspections.

- No sleeping in basement.

Entry foyer:

- There is a room just inside the front door that is not a part of any apartment.  This can be used as a community use room.  A keyed lock on the door is not allowed.

Apt 1 (1st fl):

- Entry door is not closing properly with the automatic closer.  The edge of the door is rubbing the frame.  This is a Safety Item. This must be repaired.

- Repair/replace any kitchen cabinet door that are broken or crooked.

- Poor/peeling paint observed.  This is a Safety Item. A lead inspection is required.

Apt 2 (2nd fl):

- Orphan room at top of stairs is not inside the apartment.  This is not a bedroom.

- Entry door not latching properly.  Repair.

- A latch is missing on the main entry door.  This is a Safety Item. Repair.

Apt 3 (3rd fl):

- Repair/replace kitchen cabinet doors in need.

- Peeling paint is observed in several places.  This is a Safety Item. A lead inspection is required.